# Lab 1

Computer organization and design

Taylan Unal | CMPEN 331 Section 1 | 9/22/19

# Verilog Design Code-Main (FSM)

- `timescale 1ns / 1ps

- //CMPEN 331 Lab1 Taylan Unal FA19

- module lab1( // module contains State machine.

- input u,

- input [2:0] currentState,

- input clrn,

- output reg [6:0] led,

- output reg [2:0] nextState

- );

- reg [2:0] S0 = 3'b000;

- reg [2:0] S1 = 3'b001;

- reg [2:0] S2 = 3'b010;

- reg [2:0] S3 = 3'b011;

- reg [2:0] S4 = 3'b100;

- reg [2:0] S5 = 3'b101;

- always @(*)

- begin

- case(currentState)//depending on current state, determines next state. Defined on lab sheet.

- S0: begin

- if (u == 1)

- nextState = S1;

- else

- nextState = S5;

- end

- S1: begin

- if (u == 1)

- nextState = S2;

- else

- nextState = S0;

- end

- S2: begin

- if (u == 1)

- nextState = S3;

- else

- nextState = S1;

- end

- S3: begin

- if (u == 1)

- nextState = S4;

- else

- nextState = S2;

- end

- S4: begin

- if (u == 1)

- nextState = S5;

- else

- nextState = S3;

- end

- S5: begin

- if (u == 1)

- nextState = S0;

- else

- nextState = S4;

- end

- endcase

- end

- always @(*)

- begin

- case(currentState)// letters for LED are gfedcba because bits go from [bit6 to bit0] MSB first.

- S0: begin // S0=000

- led <= 7'b1000000; //each represents the letters lit up

- end

- S1: begin // S1=001

- led <= 7'b1111001;

- end

- S2: begin // S2=010

- led <= 7'b0100100;

- end

- S3: begin // S3 = 011

- led <= 7'b0110000;

- end

- S4: begin //S4 = 100

- led <= 7'b0011001;

- end

- S5: begin //S5 = 101

- led <= 7'b0010010;

- end

- endcase

- end

- endmodule

# Verilog Test Bench Code-Main (Testbench)

- `timescale 1ns / 1ps

- module testbench;

- //INPUTS

- wire [2:0] currentState;

- reg clk;

- reg clrn;

- reg u;

- //OUTPUTS

- wire [2:0] nextState;//aka Next State

- wire [6:0] led;

- lab1 lab1(

- .u(u),

- .currentState(currentState),

- .clrn(clrn),

- .nextState(nextState),

- .led(led)

- );

- posEdgeDFF posEdgeDFF(

- .currentState(currentState),

- .clk(clk),

- .clrn(clrn),

- .nextState(nextState)//aka Next State

- );

- initial begin

- clk = 1;

- u = 1;

- clrn = 0;

- #1 clrn = 1;

- #15 u = 0;

- end

- always

- begin

- #1 clk = ~clk;

- end

- endmodule

# Verilog Design Code-DFF

- `timescale 1ns / 1ps

- module posEdgeDFF(currentState,clk,clrn,nextState);

- input [2:0] nextState; //Data input

- input clk;//clock

- input clrn;//reset

- output reg [2:0] currentState; //output Q

- always @(posedge clk or clrn)//DFF 1

- if(~clrn) begin //1bit value of 1

- currentState <= 3'b0; //1bit value of 0

- end else begin

- currentState <= nextState;

- end

- endmodule

# Verilog Testbench Code-DFF

- `timescale 1ns / 1ps

- module posEdgeDFFTest;//currentState,clk,reset,nextState

- //inputs

- reg [2:0] currentState;

- reg clk;

- reg reset;

- //output

- wire [2:0] nextState; //output Q

- posEdgeDFF uut(currentState,clk,reset,nextState);

- initial begin

- clk = 0;

- forever #10 clk = ~clk; //(flip clock every 10ns)

- end

- initial begin //testing of DFF

- reset=1;

- currentState <= 0;

- #100;

- reset=0;

- currentState <= 1;

- #100;

- currentState <= 0;

- #100;

- currentState <=1;

- end

- endmodule

# Output Waveforms

# Design Schematics

# I/O Planning

# Floor Planning