# Lab 4

Computer organization and design

Taylan Unal | CMPEN 331 Section 1 | 10/17/19

Top Module (CPU):

- //Taylan Unal CMPEN 331.001 LAB4 Top Module

- `timescale 1ns / 1ps

- module PC(clk, PCin, PCout);

- input clk;

- input [31:0] PCin;

- reg[31:0] PCmem;

- output reg [31:0] PCout;

- initial begin

- PCout = 0;

- PCmem = 100;

- end

- always @(posedge clk) begin

- PCmem <= PCin; //register output is saved as its input at clk cycle

- end

- always @(negedge clk) begin

- PCout <= PCmem; //register output is assigned its input at clk cycle

- end

- endmodule

- module PCAdder(PCin, PCout);

- input [31:0] PCin;

- output reg [31:0] PCout;

- always @(PCin) begin

- PCout <= PCin + 4;

- end

- endmodule

- module InstructMem(a, do); //a is 'PC input', do is 'IM output'

- input [31:0] a;

- output reg [31:0] do;

- reg [31:0] IM[0:511];//load word from memory, leaving room forinstructions later.

- initial begin

- IM[32'd100] = 32'h8c220000; //lw $v0 00($at)

- IM[32'd104] = 32'h8c230004; //lw $v1 04($at)

- IM[32'd108] = 32'h8c240008; //lw $a0 08($at)

- IM[32'd112] = 32'h8c25000c; //lw $a1 12($at)

- end

- always @ (a) begin

- do <= IM[a];

- end

- endmodule

- module IFID(clk, instIn, instOut);//define instruction input and instruction from IM

- input clk;

- input [31:0] instIn;

- reg [31:0] IF; //IF memory

- output reg [31:0] instOut; //defines the R-type

- always @ (posedge clk) begin

- IF <= instIn; //save value of input until negedge.

- end

- always @ (negedge clk) begin

- instOut <= IF; //pull from stored value

- end

- endmodule

- module ControlUnit(inst_CU, wreg, m2reg, wmem, aluc, aluimm, regrt);

- input [31:0] inst_CU; //ifid_out. Includes op, func.

- wire [5:0] op, func;

- output reg wreg, m2reg, wmem, aluimm, regrt;

- output reg [3:0] aluc;

- assign op = inst_CU[31:26];

- initial begin

- wreg <= 0; //RegWrite

- m2reg <= 0; //Mem2Reg

- wmem <= 0; //Write Memory

- aluimm <= 0; //ALU source

- regrt <= 0; //Reg Destination

- end

- always @ (op) begin

- if(op == 6'b100011) begin //Load Word (LW)

- wreg <= 1'b1;

- m2reg <= 1'b1;

- wmem <= 1'b0;

- aluimm <= 1'b1;

- regrt <= 1'b1; //important part

- aluc <= 4'b0010;

- end

- end

- endmodule

- module ControlMux(inst_Mux, regrt, rd_rt);

- input [31:0] inst_Mux;

- input regrt;

- output reg [4:0] rd_rt;

- wire [4:0] rd, rt;

- assign rd = inst_Mux[15:11];

- assign rt = inst_Mux[20:16];

- always @(rd, rt) begin

- case (regrt)

- 1'b1:

- rd_rt = rt;

- 1'b0:

- rd_rt = rd;

- endcase

- end

- endmodule

- module RegFile(inst_RF, qa, qb); //if_idout is inst_RF, qa is RS out, qb is RT out

- input [31:0] inst_RF;

- reg [31:0] regs [0:31]; //32 x 32 register file. Store all the registers.

- wire [4:0] rs, rt;

- output reg [31:0] qa, qb;

- assign rs = inst_RF[25:21];

- assign rt = inst_RF[20:16];

- initial begin //initialize all 32 registers to 0.

- {regs[0],regs[1],regs[2],regs[3],regs[4],regs[5],regs[6],regs[7],

- regs[8],regs[9],regs[10],regs[11],regs[12],regs[13],regs[14],regs[15],

- regs[16],regs[17],regs[18],regs[19],regs[20],regs[21],regs[22],regs[23],

- regs[24],regs[25],regs[26],regs[27],regs[28],regs[29],regs[30],regs[31]} = 0;

- end

- always @(rs, rt) begin

- qa <= regs[rs]; //register output 1 = val in register rs

- qb <= regs[rt]; //register output 2 = val in register rt

- end

- endmodule

- module SignExtend(inst_IF, immOut);//input IF instruct value, output an extended 32 bit value

- input [31:0] inst_IF; //short value

- output reg [31:0] immOut; //extended value

- wire [15:0] imm; //save wire for main values

- assign imm = inst_IF[15:0]; //save first 16 bits.

- always @(imm) begin

- immOut = {{16{imm[15]}},imm}; //extends 16bit number to 32bits.

- end

- endmodule

- module IDEXE(clk, wreg, m2reg, wmem, aluc, aluimm, mux, qa, qb, extend,

- ewreg, em2reg, ewmem, ealuc, ealuimm, emux, eqa, eqb, eextend);

- input clk;

- input wreg, m2reg, wmem, aluimm; //input to IDEXE

- input [4:0] mux; //output from mux into IDEXE

- input [3:0] aluc; //output from control unit

- input [31:0] extend, qa, qb; //output from regfile

- //Use these to store values for later assignment using posedge, negedge.

- reg wreg2, m2reg2, wmem2, aluimm2;

- reg [4:0] mux2;

- reg [3:0] aluc2;

- reg [31:0] qa2, qb2;

- reg [31:0] extend2;

- output reg ewreg, em2reg, ewmem, ealuimm; //extended outputs from control unit

- output reg [3:0] ealuc; //extended outputs from control unit, into ALU

- output reg [4:0] emux; //extended outputs from multiplexer

- output reg [31:0] eqa, eqb; //extended outputs from regfile

- output reg [31:0] eextend; //extended outputs from sign extender

- always@(posedge clk) begin //pass values into middle save values. (save regs <= input)

- wreg2 <= wreg;

- m2reg2 <= m2reg;

- wmem2 <= wmem;

- aluimm2 <= aluimm;

- mux2 <= mux;

- aluc2 <= aluc;

- qa2 <= qa;

- qb2 <= qb;

- extend2 <= extend;

- end

- always@(negedge clk) begin //output values from saved values. (output <= save regs)

- ewreg <= wreg2;

- em2reg <= m2reg2;

- ewmem <= wmem2;

- ealuimm <= aluimm2;

- emux <= mux2;

- ealuc <= aluc2;

- eqa <= qa2;

- eqb <= qb2;

- eextend <= extend2;

- end

- endmodule

- module ALUMux(ealuimm, eqb, eextend, alumux_out);

- input ealuimm;

- input [31:0] eqb, eextend; //qb value and immExtended

- output reg [31:0] alumux_out;

- always @ (eqb, eextend) begin

- case (ealuimm)

- 1'b0: alumux_out = eqb;

- 1'b1: alumux_out = eextend;

- endcase

- end

- endmodule

- module ALU (ALUcontrol, eqa , eqb, alu_out);

- input [3:0] ALUcontrol; //4bit number

- input [31:0] eqa, eqb;

- output reg [31:0] alu_out;

- always @ (eqa, eqb) begin

- case (ALUcontrol)

- 4'b0010: alu_out <= eqa + eqb;

- endcase

- end

- endmodule

- module EXEMEM (clock, eWREG, eM2REG, eWMEM, eMUX, eALU_OUT, eQB, eWREG_out,

- eM2REG_out, eWMEM_out, eMUX_out, eALU_out, eQB_out);

- input clock, eWREG, eM2REG, eWMEM;

- input [4:0] eMUX;

- input [31:0] eQB, eALU_OUT;

- reg ewreg, em2reg, ewmem;

- reg [4:0] emux;

- reg [31:0] ealu_out, eqb;

- output reg eWREG_out, eM2REG_out, eWMEM_out;

- output reg [4:0] eMUX_out;

- output reg [31:0] eALU_out, eQB_out;

- always @ (posedge clock) begin

- ewreg <= eWREG;

- em2reg <= eM2REG;

- ewmem <= eWMEM;

- emux <= eMUX;

- ealu_out <= eALU_OUT;

- eqb <= eQB;

- end

- always @ (negedge clock) begin

- eWREG_out <= ewreg;

- eM2REG_out <= em2reg;

- eWMEM_out <= ewmem;

- eMUX_out <= emux;

- eALU_out <= ealu_out;

- eQB_out <= eqb;

- end

- endmodule

- module DataMemory (MEMWRITE, ALUDATA_IN ,QBDATA_IN, DMDATA_OUT);

- input MEMWRITE;

- input [31:0] ALUDATA_IN, QBDATA_IN;

- reg [31:0] DM [0:36];

- output reg [31:0] DMDATA_OUT;

- initial begin //set first 10 words to data memory

- DM[32'd0] = 32'hA00000AA;

- DM[32'd4] = 32'h10000011;

- DM[32'd8] = 32'h20000022;

- DM[32'd12] = 32'h30000033;

- DM[32'd16] = 32'h40000044;

- DM[32'd20] = 32'h50000055;

- DM[32'd24] = 32'h60000066;

- DM[32'd28] = 32'h70000077;

- DM[32'd32] = 32'h80000088;

- DM[32'd36] = 32'h90000099;

- end

- always @ (ALUDATA_IN, QBDATA_IN) begin

- case (MEMWRITE)

- 1'b0: DMDATA_OUT <= DM[ALUDATA_IN];

- 1'b1: DMDATA_OUT <= DM[QBDATA_IN];

- endcase

- end

- endmodule

- module MemWB(clock, mWREG, mM2REG, mMUX, mALU, mDM);

- input clock, mWREG, mM2REG;

- input [4:0] mMUX;

- input [31:0] mALU, mDM;

- reg mwreg,mm2reg;

- reg [4:0] mmux;

- reg [31:0] malu,mdm;

- always @(posedge clock) begin

- mwreg <= mWREG;

- mm2reg <= mM2REG;

- mmux <= mMUX;

- malu <= mALU;

- mdm <= mDM;

- end

- endmodule

Testbench Module:

- //Taylan Unal CMPEN 331.001 LAB4 Testbench

- `timescale 1ns / 1ps

- module CPU_Test();

- //WIRES ARE OUTPUTS, REGS ARE INPUTS

- reg clk; //could be clk = 0;

- wire [31:0] pcin, pcout, im_out,if_idout,qawire,qbwire,sign_extendedout, eqawire

- eqbwire, eimmnext_out, alumux_out, alu_out, e2mALU, qb2DM, dm_out;

- wire [3:0] aluc_out, ealuc_out;

- wire wregwire, m2regwire, wmemwire, aluimmwire, regrtwire, ewregwire,

- em2regwire, ewmemwire, ealuimmwire, e2mwreg, e2mM2reg, wmemDM;

- wire [4:0] muxwire, emuxwire, e2mMux; //e2mMux;

- //////////////////////////////////////////////////////////////////////////////////////

- PC PC(clk, pcin, pcout); //done

- PCAdder adder(pcout, pcin); //done

- InstructMem instmem(pcout, im_out); //done

- IFID ifid(clk, im_out, if_idout);//needs clk, im_out is Ifid_in, if_idout is output

- ControlUnit ctrunit(if_idout, wregwire, m2regwire, wmemwire,

- aluc_out, aluimmwire, regrtwire);

- ControlMux ctrmux(if_idout, regrtwire, muxwire); // inst_Mux contains ifid, regrt from control unit, muxwire contains rd_rt

- RegFile regfile(if_idout, qawire, qbwire); //if_id is inst_RF, qawire is RS output, qbwire is RT output

- SignExtend extender(if_idout, sign_extendedout); //input 32 bit non sign extended, output 32 bit extended.

- IDEXE idexe(clk,wregwire,m2regwire,wmemwire, aluc_out, aluimmwire, muxwire, qawire, qbwire, sign_extendedout, ewregwire, em2regwire, ewmemwire, ealuc_out, ealuimmwire,

- emuxwire, eqawire, eqbwire, eimmnext_out); //working on it

- ALUMux alumux(ealuimmwire, eqbwire, eimmnext_out, alumux_out);

- ALU alu(ealuc_out, eqawire, alumux_out, alu_out);

- EXEMEM exemem(clk, ewregwire, em2regwire, ewmemwire, emuxwire, alu_out, eqbwire,

- e2mwreg, e2mM2reg, wmemDM, e2mMux, e2mALU, qb2DM);

- DataMemory datamem(wmemDM, e2mALU, qb2DM, dm_out); //memwrite, alu input, qbdata input, DM output

- MemWB memwb(clk, e2mwreg, e2mM2reg, e2mMux, e2mALU, dm_out); //clk, memwrite reg, mem Mux, mem ALU, mem DM

- initial begin

- clk = 0;

- end

- always begin

- #5 clk = !clk;

- end

- endmodule

Waveforms:

Design Schematic:

Design Schematic (contd):

Design Schematic (contd):

IO Planning:

Floor Planning: