# Lab 5

Computer organization and design

Taylan Unal | CMPEN 331 Section 1 | 10/17/19

Top Module (CPU):

- //Taylan Unal CMPEN 331.001 LAB5 Top Module

- `timescale 1ns / 1ps

- module PC(clk, PCin, PCout);

- input clk;

- input [31:0] PCin;

- reg[31:0] PCmem;

- output reg [31:0] PCout;

- initial begin

- PCout = 0;

- PCmem = 100;

- end

- always @(posedge clk) begin

- PCmem <= PCin; //register output is saved as its input at clk cycle

- end

- always @(negedge clk) begin

- PCout <= PCmem; //register output is assigned its input at clk cycle

- end

- endmodule

- module PCAdder(PCin, PCout);

- input [31:0] PCin;

- output reg [31:0] PCout;

- always @(PCin) begin

- PCout <= PCin + 4;

- end

- endmodule

- module InstMem(pcin, do); //a is 'PC input', do is 'IM output'

- input [31:0] pcin;

- output reg [31:0] do;

- reg [31:0] IM[0:511]; //load word from memory, leaving room for more instructions later.

- initial begin

- IM[32'd100] = 32'h8c220000; //lw $v0 00($at)

- IM[32'd104] = 32'h8c230004; //lw $v1 04($at)

- IM[32'd108] = 32'h8c240008; //lw $a0 08($at)

- IM[32'd112] = 32'h8c25000c; //lw $a1 12($at)

- IM[32'd116] = 32'h012E6820; //add $t5,$1,$t6 (adding $6,$2,$10)

- //IM[32'd116] = 32'h004a3020; //add $a2,$v0,$t2 (Jerome)

- end

- always @ (pcin) begin //good

- do <= IM[pcin];

- end

- endmodule

- module IFID(clk, do, op, rd, rs, rt, func, imm);

- input clk;

- input [31:0] do;//instruction input

- output reg [5:0] op,func;//I-type values

- output reg [5:0] rd,rs,rt;//I-type values

- output reg [15:0] imm;//immediate value

- reg [31:0] IF; //IFID  temporarymemory

- always @ (posedge clk) begin

- IF <= do; //save value of input until negedge

- end

- always @(negedge clk)

- begin

- op <= IF[31:26];

- rs <= IF[25:21];

- rt <= IF[20:16];

- rd <= IF[15:11];

- func <= IF[5:0];

- imm <= IF[15:0]; //hmmm

- end

- endmodule

- module ControlUnit(op,func,aluc,wreg,m2reg,wmem,aluimm,regrt);

- input [5:0] op,func;

- output reg [3:0] aluc;

- output reg wreg, m2reg, wmem, aluimm, regrt;

- initial begin

- wreg <= 0; //RegWrite

- m2reg <= 0; //Mem2Reg

- wmem <= 0; //Write Memory

- aluimm <= 0; //ALU source

- regrt <= 0; //Reg Destination

- end

- always @ (op,func) begin //adding func

- if(op == 0) begin //R-Type instruction

- case(func)

- 32: aluc <= 2; //Register ADD

- 34: aluc <= 6; //Register SUB

- 36: aluc <= 0; //Register AND

- 37: aluc <= 1; //Register OR

- 39: aluc <= 12;//Register XOR

- endcase

- wreg <= 1;//RegWrite

- m2reg <= 0;//Mem2Reg

- wmem <= 0;//WriteMem

- aluimm <= 0; //AluSrc

- regrt <= 1; //input to ControlMux

- end

- if(op == 6'b100011) begin //Load Word (LW) I-type

- aluc <= 2;//add

- wreg <= 1;//RegWrite

- m2reg <= 1;//Mem2Reg

- wmem <= 0;//WriteMem

- aluimm <= 1;//AluSrc

- regrt <= 0; //input to ControlMux

- end

- end

- endmodule

- module ControlMux(rd,rt,regrt,rn);

- input [4:0] rd,rt; //output from IFIDReg

- input regrt; //from controlunit, enable write

- output reg [4:0] rn; //output

- always @(regrt,rt,rd) begin

- case (regrt)

- 0:

- rn <= rt;

- 1:

- rn <= rd;

- endcase

- end

- endmodule

- module SignExtend(immIn, immOut);//input IF instruct value, output an extended 32 bit value

- input [15:0] immIn; //short (nonextended) value

- output reg [31:0] immOut; //extended value

- always @(immIn) begin

- immOut <= {{16{immIn[15]}},immIn[15:0]}; //extends 16bit number to 32bits.

- end

- endmodule

- module IDEXE(clk, wreg, m2reg, wmem, aluc, aluimm, rn, qa, qb, imm,

- ewreg, em2reg, ewmem, ealuc, ealuimm, ern, eqa, eqb, eimm);

- input clk;

- input wreg, m2reg, wmem, aluimm; //input to IDEXE

- input [3:0] aluc; //output from control unit

- input [4:0] rn; //output from mux into IDEXE

- input [31:0] qa, qb; //output from regfile

- input [31:0] imm; //extended immediate value

- //Use these to store values for later assignment using posedge, negedge.

- reg wreg2, m2reg2, wmem2, aluimm2;

- reg [4:0] rn2;

- reg [3:0] aluc2;

- reg [31:0] qa2, qb2;

- reg [31:0] imm2;

- output reg ewreg, em2reg, ewmem, ealuimm; //extended outputs from control unit

- output reg [3:0] ealuc; //extended outputs from control unit, into ALU

- output reg [4:0] ern; //extended outputs from mux

- output reg [31:0] eqa, eqb; //extended outputs from regfile

- output reg [31:0] eimm; //extended outputs from sign extender

- always@(posedge clk) begin //pass values into middle save values.(save regs<=input)

- wreg2 <= wreg;

- m2reg2 <= m2reg;

- wmem2 <= wmem;

- aluimm2 <= aluimm;

- rn2 <= rn;

- aluc2 <= aluc;

- qa2 <= qa;

- qb2 <= qb;

- imm2 <= imm;

- end

- always@(negedge clk) begin //output values from saved values. (output <= save regs)

- ewreg <= wreg2;

- em2reg <= m2reg2;

- ewmem <= wmem2;

- ealuimm <= aluimm2;

- ern <= rn2;

- ealuc <= aluc2;

- eqa <= qa2;

- eqb <= qb2;

- eimm <= imm2;

- end

- endmodule

- module ALUMux(eqb, eimmExt, ealuimm, muxImm);

- input [31:0] eqb, eimmExt; //qb value and immExt from IDEXE

- input ealuimm;//selector from IDEXE

- output reg [31:0] muxImm; //output of ALUMux

- always @ (eqb, eimmExt) begin

- case (ealuimm)

- 0: muxImm = eqb;

- 1: muxImm = eimmExt;

- endcase

- end

- endmodule

- module ALU (eqa, eqb, ealuc, aluOut);

- input [31:0] eqa, eqb;

- input [3:0] ealuc; //4bit number

- output reg [31:0] aluOut;

- always @ (eqa, eqb) begin

- case (ealuc)

- 4'b0010: aluOut <= eqa + eqb;

- endcase

- end

- endmodule

- module EXEMEM (clock, ewreg, em2reg, ewmem, ern, aluOut, eqb,

- mwreg, mm2reg, mwmem, mrn, maluOut, mqb);

- input clock, ewreg, em2reg, ewmem;

- input [4:0] ern;

- input [31:0] aluOut, eqb;

- //Temporary store values for posedge/negedge

- reg ewreg2, em2reg2, ewmem2;

- reg [4:0] ern2;

- reg [31:0] aluOut2, eqb2;

- output reg mwreg, mm2reg, mwmem; //outputs on memory side of register

- output reg [4:0] mrn;

- output reg [31:0] maluOut, mqb;

- always @ (posedge clock) begin //pass values into middle save (save regs <= input)

- ewreg2 <= ewreg;

- em2reg2 <= em2reg;

- ewmem2 <= ewmem;

- ern2 <= ern;

- aluOut2 <= aluOut;

- eqb2 <= eqb;

- end

- always @ (negedge clock) begin

- mwreg <= ewreg2;

- mm2reg <= em2reg;

- mwmem <= ewmem;

- mrn <= ern2;

- maluOut <= aluOut2;

- mqb <= eqb2;

- end

- endmodule

- module DataMemory (maluOut, mqb, mwmem, dmOut);

- input [31:0] maluOut, mqb; //input alu output value, qb value

- input mwmem;//write memory

- output reg [31:0] dmOut;

- reg [31:0] DM [0:36];

- initial begin //set first 10 words to data memory

- DM[32'd0] = 32'hA00000AA;

- DM[32'd4] = 32'h10000011;

- DM[32'd8] = 32'h20000022;

- DM[32'd12] = 32'h30000033;

- DM[32'd16] = 32'h40000044;

- DM[32'd20] = 32'h50000055;

- DM[32'd24] = 32'h60000066;

- DM[32'd28] = 32'h70000077;

- DM[32'd32] = 32'h80000088;

- DM[32'd36] = 32'h90000099;

- end

- always @ (maluOut, mqb) begin

- case (mwmem)

- 1'b0: dmOut <= DM[maluOut];

- 1'b1: dmOut <= DM[mqb];

- endcase

- end

- endmodule

- module MEMWB(clk, mwreg, mm2reg, mrn, maluOut, dmOut,

- wwreg, wm2reg, wrn, waluOut, wdmOut);//adding WB outputs

- input clk, mwreg, mm2reg;

- input [31:0] maluOut, dmOut;

- input [4:0] mrn;

- //Temporary store values between posedge/negedge

- reg mwreg2, mm2reg2;

- reg [31:0] maluOut2, dmOut2;

- reg [4:0] mrn2;

- output reg wwreg, wm2reg;

- output reg [31:0] waluOut, wdmOut;

- output reg [4:0] wrn;

- always @(posedge clk) begin //stores into temporary regs

- mwreg2 <= mwreg;

- mm2reg2 <= mm2reg;

- maluOut2 <= maluOut;

- dmOut2 <= dmOut;

- mrn2 <= mrn;

- end

- always @(negedge clk) begin

- wwreg <= mwreg2;

- wm2reg <= mm2reg2;

- waluOut <= maluOut2;

- wdmOut <= dmOut2;

- wrn <= mrn2;

- end

- endmodule

- module WBMux(waluOut, wdmOut, wm2reg, wbMuxOut);

- input [31:0] waluOut, wdmOut;//takes in aluresult and data memory address

- input wm2reg; //write enable memory to register

- output reg [31:0] wbMuxOut; //output to regfile

- always @(waluOut, wdmOut, wm2reg) begin

- case(wm2reg)

- 0: wbMuxOut <= waluOut; //if wm2reg=0, output alu value

- 1: wbMuxOut <= wdmOut; //if wm2reg=1, output datamem value.

- endcase

- end

- endmodule

- module RegFile(clk, we, rs, rt, qa, qb, wrn, wbmuxOut); //(clk,rs,rt,qa,qb,wrn,wbmuxOut)

- input clk;

- input we; //write enable input (wwreg

- input [31:0] wbmuxOut; //data to write to register

- input [4:0] rs, rt, wrn; //rs val in, rt val in, rd val in

- reg [31:0] regs [0:31]; //32 x 32 register file. Store all the registers.

- output reg [31:0] qa, qb; //qa->rt out, qb->rs

- initial begin //initialize all 32 registers to 0.

- {regs[0], regs[1], regs[2], regs[3], regs[4], regs[5], regs[6], regs[7],

- regs[8], regs[9], regs[10], regs[11], regs[12], regs[13], regs[14], regs[15],

- regs[16], regs[17], regs[18], regs[19], regs[20], regs[21], regs[22], regs[23],

- regs[24], regs[25], regs[26], regs[27], regs[28], regs[29], regs[30], regs[31]} = 0;//32'h00000000;

- end

- always @(posedge clk) begin //write during first half of cycle

- if(wrn != 0) begin//write operation

- regs[wrn] <= wbmuxOut;

- end

- end

- always @(rs, rt) begin//read during second half of cycle

- qa <= regs[rs]; //register val in register rs

- qb <= regs[rt]; //register val in register rt

- end

- endmodule

Testbench Module:

- //Taylan Unal CMPEN 331.001 LAB5 Testbench

- `timescale 1ns / 1ps

- module CPU_Test();

- //WIRES ARE OUTPUTS, REGS ARE INPUTS.

- reg clk; //global clock signal

- //IF STAGE

- wire [31:0] pcin; //PC output to PC register

- wire [31:0] pcout; //PC output to adder

- wire [31:0] do; //instruction to be executed (im_out)

- //ID STAGE

- wire [5:0] op, func; //opcode and func fields.

- wire [4:0] rd,rs,rt; //source, destination registers of instruction

- wire [15:0] imm; //immediate field (for i-type instructions)

- wire [3:0] aluc; //alucontrol to input into ALU to determine operation

- wire wreg; //(ID) write enable for IDEXE register

- wire m2reg; //(ID) write enable for IDEXE register from data memory

- wire wmem; //(ID) write enable for data memory

- wire aluimm; //(ID) sends either alu control signal or immediate value to ALUMUX

- wire regrt; // (ID) input to controlMux, chooses what to write rt/rd to IDEXEregister

- wire [4:0] rn; //(ID) ** output of controlMux, chooses to write reg number to IDEXE

- wire [31:0] qa,qb;//(ID) ** register file values for instruction. (Note register is now at end of WB)

- wire [31:0] immExt; // (ID)extended immediate value to 32 bits (I-type)

- //EXE STAGE

- wire [3:0] ealuc; //(EXE) ALUControl to determine ALU operation

- wire ewreg; //(EXE) write enable for EXEMEM register

- wire em2reg; //(EXE) write enable for EXEMEM register from data memory

- wire ewmem; //(EXE) write enable for data memory

- wire ealuimm; //(EXE) sends either ALUcontrol signal or immediate value to ALUMUX

- wire [4:0] ern; //(EXE) **output of controlMux, chooses to write reg number to EXEMEM

- wire [31:0] eqa, eqb; //(EXE) **register file values for instruction

- wire [31:0] eimmExt; //(EXE) extended immediate value after written to IDEXE

- wire [31:0] muxImm; //(EXE) output of immediate mux

- wire [31:0] aluOut; //(EXE) output of ALU Module

- //MEM STAGE

- wire mwreg; //(MEM) write enable for MEMWB register

- wire mm2reg; //(MEM) write enable for MEMWB register from data memory

- wire mwmem; //(MEM) write enable for data memory

- wire [4:0] mrn; //(MEM) **output of controlMux, chooses to write reg number to MEMWB

- wire [31:0] maluOut; //(MEM) output of ALU Module

- wire [31:0] mqb; //(MEM) qb value from regfile->IDEXEReg->EXEMEMReg

- wire [31:0] dmOut; //(MEM) data memory output

- //WB STAGE

- wire wwreg; //(WB) write enable for Regfile. (After all other stages enable values

- wire wm2reg; //(WB) write enable for Regfile from datamemory

- wire [4:0] wrn; //(WB) **output of controlMux, chooses to write reg number to Regfile

- wire [31:0] waluOut; //(WB) output of ALU Module

- wire [31:0] wdmOut; //(WB) data memory output

- wire [31:0] wbMuxOut;//(WB) output of Mem2Reg Mux for DataMem->Regfile

- //////////////////////////////////////////////////////////////////////////////////////

- //START Testbench

- //IF STAGE

- PC PC(clk, pcin, pcout);//PCRegister: input clk, PCinput, outputs result from PCAdder

- PCAdder adder(pcout, pcin);//PCAdder: input PCOutput, increments by 4, outputs to input or

- InstMem instmem(pcout,do);//InstMem: has instructions, outputs instruction from PCRegister

- //IFID REGISTER

- IFID ifid(clk,do,op,rd,rs,rt,func,imm);

- //ID STAGE

- ControlUnit CU(op,func,aluc,wreg,m2reg,wmem,aluimm,regrt); //ControlUnit: inputs op,func fields, outputs aluc,wreg,m2reg,wmem,aluimm into IDEXEReg and regrt into RegisterMux

- ControlMux ctrmux(rd,rt,regrt,rn); //ControlMux: Selects whether to write rd/rt into RegWrite, outputs to IDEXE

- SignExtend extender(imm, immExt); //SignExtend: input is 16bit nonextended from instruction memory, output 32bit extended value.

- //IDEXE REGISTER

- IDEXE idexe(clk,wreg,m2reg,wmem,aluc,aluimm,rn,qa,qb,immExt,//inputs

- ewreg,em2reg,ewmem,ealuc,ealuimm,ern,eqa,eqb,eimmExt);//outputs

- //EXE STAGE

- ALUMux alumux(eqb,eimmExt,ealuimm,muxImm);//ALUMux: selects qb reg output or immediate value

- ALU alu(eqa, muxImm, ealuc, aluOut);//ALU: executes operations for CPU using aluc control, a,b inputs

- //EXEMEM REGISTER

- EXEMEM exemem(clk,ewreg,em2reg,ewmem,ern,aluOut,eqb,//inputs

- mwreg,mm2reg,mwmem,mrn,maluOut,mqb);//outputs

- //MEM STAGE

- DataMemory datamem(maluOut,mqb,mwmem,dmOut); //DataMemory: data memory handler, inputs from EXEMEM, out to dmOut

- //MEMWB REGISTER

- MEMWB memwb(clk,mwreg,mm2reg,mrn,maluOut,dmOut,//inputs

- wwreg,wm2reg,wrn,waluOut,wdmOut); //ouputs

- //WB STAGE

- WBMux wbmux(waluOut, wdmOut, wm2reg, wbMuxOut); //WBMux: selects what to write back into regfile

- RegFile regfile(clk,rs,rt,qa,qb,wrn,wbMuxOut); //RegFile: generates a 32x32 register file to read/write from

- initial begin //Clock signal loop.

- clk = 0;

- end

- always begin

- #5 clk = !clk;

- end

- endmodule

Waveforms:

Design Schematic:

Design Schematic (contd):

Design Schematic (contd):

IO Planning:

Floor Planning: