# Lab 5 (Honors Option)

Computer organization and design

Taylan Unal | CMPEN 331 Section 1 | 12/21/19

Summary:

Throughout the semester, stage by stage, we’ve worked towards implementing a fully functioning CPU design over on FPGA (XC7Z010--1CLG400C). The design features 5 stages (IF, ID, EXE, MEM, WB) complete with registers (IFID, IDEXE, EXEMEM, MEMWB) and various components needed by the CPU to perform necessary functions. The CPU features forwarding and pipelining to avoid hazards common to instruction sets requiring the same data values. Additionally, the CPU can handle most common instructions like loading, storing data to registers/memory, performing arithmetic operations on register values and memory values, and can handle branches and jumps in instructions. Working through this lab, I learned a great deal about the intricacies of designing a functioning CPU, correctly defining variables and I/O, and ensuring designing an accurate testbench.

Top Module (CPU):

- //Taylan Unal CMPEN 331.001 Lab5 Honors Option Top Module

- //Avoiding Data Hazards in CPU

- `timescale 1ns / 1ps

- module PC(clk, PCin, PCout);

- input clk;

- input [31:0] PCin;

- output reg [31:0] PCout;

- initial begin

- PCout = 100;

- end

- always @(posedge clk) begin

- PCout <= PCin;

- end

- endmodule

- module PCAdder(PCin, PCout);

- input [31:0] PCin;

- output reg [31:0] PCout;

- always @(PCin) begin

- PCout <= PCin + 4;

- end

- endmodule

- module InstMem(pcin, do); //a is 'PC input', do is 'IM output'

- input [31:0] pcin;

- output reg [31:0] do;

- reg [31:0] IM[0:127]; //load word from memory, leaving room for more instructions later.

- initial begin

- IM[32'd100] = 32'h8c220000; //lw $v0 00($at)

- IM[32'd104] = 32'h8c230004; //lw $v1 04($at)

- IM[32'd108] = 32'h8c240008; //lw $a0 08($at)

- IM[32'd112] = 32'h8c25000c; //lw $a1 12($at)

- IM[32'd116] = 32'h012E6820; //add $t5,$1,$t6 (adding $6,$2,$10)

- end

- always @ (pcin) begin //good

- do <= IM[pcin];

- end

- endmodule

- module IFID(clk, do, op, rd, rs, rt, func, imm);

- input clk;

- input [31:0] do;//instruction input

- output reg [5:0] op,func;//I-type values

- output reg [5:0] rd,rs,rt;//I-type values

- output reg [15:0] imm;//immediate value

- reg [31:0] IF; //IFID  temporarymemory

- always @ (posedge clk) begin

- IF <= do; //save value of input until negedge

- end

- always @(negedge clk)

- begin

- op <= IF[31:26];

- rs <= IF[25:21];

- rt <= IF[20:16];

- rd <= IF[15:11];

- func <= IF[5:0];

- imm <= IF[15:0];

- end

- endmodule

- //UPDATED ControlUnit with internal forwarding and pipeline stall.

- //Have to extend CU to detect and handle hazards.

- module ControlUnit(op,func,rs,rt,mrn,mm2reg,mwreg,ern,em2reg,ewreg, //inputs

- aluc,wreg,m2reg,wmem,aluimm,fwda,fwdb,wpcir,regrt);//outputs

- input [5:0] op,func;

- input [4:0] rs,rt; //added rs,rt to control unit.

- input [4:0] mrn; // MEM RN

- input mm2reg,mwreg; // MEM m2reg, wreg

- input [4:0] ern; // EXE RN

- input em2reg,ewreg; // EXE m2reg, wreg

- output reg [3:0] aluc;

- output reg wreg, m2reg, wmem, aluimm, regrt;

- output reg [1:0] fwda, fwdb; //Forwarding values to MUX.

- output reg wpcir; //Write enable for PC

- initial begin

- wreg <= 0; //RegWrite

- m2reg <= 0; //Mem2Reg

- wmem <= 0; //Write Memory

- aluimm <= 0; //ALU source

- regrt <= 0; //Reg Destination

- end

- always @ (op,func) begin //adding func

- if(op == 0) begin //R-Type instructions

- case(func)

- 32: aluc <= 2; //Register ADD

- 34: aluc <= 6; //Register SUB

- 36: aluc <= 0; //Register AND

- 37: aluc <= 1; //Register OR

- 39: aluc <= 12;//Register XOR

- endcase

- wreg <= 1;//RegWrite

- m2reg <= 0;//Mem2Reg

- wmem <= 0;//WriteMem

- aluimm <= 0; //AluSrc

- regrt <= 1; //input to ControlMux

- end

- if(op == 6'b100011) begin //Load Word (LW) I-type

- aluc <= 2;//add

- wreg <= 1;//RegWrite

- m2reg <= 1;//Mem2Reg

- wmem <= 0;//WriteMem

- aluimm <= 1;//AluSrc

- regrt <= 0; //input to ControlMux

- end

- if(op == 6'b101011) begin //Store Word (SW) I-type

- aluc <= 2; //add

- wreg <= 0;//RegWrite

- m2reg <= 0;//Mem2Reg

- wmem <= 1;//WriteMem

- aluimm <= 1;//AluSrc

- regrt <= 1; //input to ControlMux

- end

- if (op == 6'b000100) //Branch Equals (BEQ)

- begin

- aluc <= 6; //subtract

- wreg <= 0; //WriteReg

- m2reg <= 0;//Mem2Reg

- wmem <= 0; //Write Memory

- aluimm <= 0; //ALUsrc

- regrt <= 1; //input to ControlMux

- end

- end

- //Start Forwarding Unit. From Textbook pg. 222

- //Inputs: rs,rt,mrn,mm2reg,mwreg,ern,em2reg,ewreg. Outputs: fwda,fwdb

- always @ (rs or rt) begin //input RS and RT into CU.

- //Forward A Register

- if (ewreg && (ern != 0) && !em2reg) begin

- assign fwda = 2'b01; //MEM Hazard. Select EXE_ALU (removing assign)

- end

- else if (mwreg && (mrn != 0) && !mm2reg) begin

- assign fwda = 2'b10; //EXE Hazard. Select MEM_ALU

- end

- else if (mwreg && (mrn != 0) && mm2reg) begin

- assign fwda = 2'b11; //EXE Hazard. Select MEM_LW

- end

- else begin

- assign fwda = 2'b00;//default, no hazards

- end

- //Forward B Register

- if (ewreg && (ern != 0) && !em2reg) begin

- assign fwdb = 2'b01; //MEM Hazard. Select maluOut

- end

- else if (mwreg && (mrn != 0) && !mm2reg) begin

- assign fwdb = 2'b10; //EXE Hazard. Select mqb

- end

- else if (mwreg && (mrn != 0) && mm2reg) begin

- assign fwdb = 2'b11; //EXE Hazard. Select dmOut

- end

- else begin

- assign fwdb = 2'b00;//default, no hazards

- end

- end

- endmodule

- module ForwardAMux(fwda,qa,maluOut,mqb,dmOut,fwda_out);

- //input forwardingA [1:0]

- //input 0: qa [31:0], 1: maluOut [31:0], 2: mqb [31:0], 3: dmOut [31:0]

- //output: FWDA_out [31:0]. to IDEXE, then is (a) value in ALU

- input [1:0] fwda; //fwda signal from CU

- input [31:0] qa, maluOut, mqb, dmOut; //WB signals: (qa), MEM:(maluOut,mqb,dmOut)

- output fwda_out;

- reg [31:0] fwda_out;

- always @ (qa or maluOut or mqb or dmOut or fwda)//if selector, any signal enabled

- begin

- case (fwda)//depending on fwda state, select output

- 2'b00: fwda_out <= qa;//means no hazard, go to regfile (qa)

- 2'b01: fwda_out <= maluOut;//MEM Hazard. Select maluOut

- 2'b10: fwda_out <= mqb;//EXE Hazard. Select mqb

- 2'b11: fwda_out <= dmOut;//EXE Hazard. Select dmOut

- endcase

- end

- endmodule

- module ForwardBMux(fwdb,qb,maluOut,mqb,dmOut,fwdb_out);

- //input forwardingB [1:0]

- //input 0: qb, 1: maluOut [31:0], 2: mqb [31:0], 3: dmOut [31:0]

- //output: FWDB_out [31:0]. to IDEXE, goes to eqb in ALUMUX.

- input [1:0] fwdb; //fwda signal from CU

- input [31:0] qb, maluOut, mqb, dmOut; //WB signals: (qa), MEM:(maluOut,mqb,dmOut)

- output fwdb_out;

- reg [31:0] fwdb_out;

- always @ (qb or maluOut or mqb or dmOut or fwdb)//if selector, any signal enabled

- begin

- case (fwdb)//depending on fwda state, select output

- 2'b00: fwdb_out <= qb;//means no hazard, go to regfile (qa)

- 2'b01: fwdb_out <= maluOut;//MEM Hazard. Select maluOut

- 2'b10: fwdb_out <= mqb;//EXE Hazard. Select mqb

- 2'b11: fwdb_out <= dmOut;//EXE Hazard. Select dmOut

- endcase

- end

- endmodule

- module ControlMux(rd,rt,regrt,rn); //selects either rd,rt

- input [4:0] rd,rt; //output from IFIDReg

- input regrt; //from controlunit, enable write

- output reg [4:0] rn; //output

- always @(regrt,rt,rd) begin

- case (regrt)

- 0:

- rn <= rt;

- 1:

- rn <= rd;

- endcase

- end

- endmodule

- module SignExtend(immIn, immOut);//input IF instruct val, output extend 32 bit val

- input [15:0] immIn; //short (nonextended) value

- output reg [31:0] immOut; //extended value

- always @(immIn) begin

- immOut <= {{16{immIn[15]}},immIn[15:0]}; //extends 16bit number to 32bits.

- end

- endmodule

- module IDEXE(clk, wreg, m2reg, wmem, aluc, aluimm, rn, qa, qb, imm,

- ewreg, em2reg, ewmem, ealuc, ealuimm, ern, eqa, eqb, eimm);

- input clk;

- input wreg, m2reg, wmem, aluimm; //input to IDEXE

- input [3:0] aluc; //output from control unit

- input [4:0] rn; //output from mux into IDEXE

- input [31:0] qa, qb; //output from regfile

- input [31:0] imm; //extended immediate value

- //Use these to store values for later assignment using posedge, negedge.

- reg wreg2, m2reg2, wmem2, aluimm2;

- reg [4:0] rn2;

- reg [3:0] aluc2;

- reg [31:0] qa2, qb2;

- reg [31:0] imm2;

- output reg ewreg, em2reg, ewmem, ealuimm; //extended outputs from control unit

- output reg [3:0] ealuc; //extended outputs from control unit, into ALU

- output reg [4:0] ern; //extended outputs from mux

- output reg [31:0] eqa, eqb; //extended outputs from regfile

- output reg [31:0] eimm; //extended outputs from sign extender

- always@(posedge clk) begin //pass values into middle save values.(save regs<=input)

- wreg2 <= wreg;

- m2reg2 <= m2reg;

- wmem2 <= wmem;

- aluimm2 <= aluimm;

- rn2 <= rn;

- aluc2 <= aluc;

- qa2 <= qa;

- qb2 <= qb;

- imm2 <= imm;

- end

- always@(negedge clk) begin //output values from saved values. (output <= save regs)

- ewreg <= wreg2;

- em2reg <= m2reg2;

- ewmem <= wmem2;

- ealuimm <= aluimm2;

- ern <= rn2;

- ealuc <= aluc2;

- eqa <= qa2;

- eqb <= qb2;

- eimm <= imm2;

- end

- endmodule

- module ALUMux(eqb, eimmExt, ealuimm, muxImm);

- input [31:0] eqb, eimmExt; //qb value and immExt from IDEXE

- input ealuimm;//selector from IDEXE

- output reg [31:0] muxImm; //output of ALUMux

- always @ (eqb, eimmExt) begin

- case (ealuimm)

- 0: muxImm = eqb;

- 1: muxImm = eimmExt;

- endcase

- end

- endmodule

- module ALU (eqa, eqb, ealuc, aluOut);

- input [31:0] eqa, eqb;

- input [3:0] ealuc; //4bit number

- output reg [31:0] aluOut;

- always @ (eqa, eqb) begin

- case (ealuc)

- 4'b0010: aluOut <= eqa + eqb;

- endcase

- end

- endmodule

- module EXEMEM (clock, ewreg, em2reg, ewmem, ern, aluOut, eqb,

- mwreg, mm2reg, mwmem, mrn, maluOut, mqb);

- input clock, ewreg, em2reg, ewmem;

- input [4:0] ern;

- input [31:0] aluOut, eqb;

- reg ewreg2, em2reg2, ewmem2;

- reg [4:0] ern2;

- reg [31:0] aluOut2, eqb2;

- output reg mwreg, mm2reg, mwmem; //outputs on memory side of register

- output reg [4:0] mrn;

- output reg [31:0] maluOut, mqb;

- always @ (posedge clock) begin //pass values into middle save (save regs <= input)

- ewreg2 <= ewreg;

- em2reg2 <= em2reg;

- ewmem2 <= ewmem;

- ern2 <= ern;

- aluOut2 <= aluOut;

- eqb2 <= eqb;

- end

- always @ (negedge clock) begin

- mwreg <= ewreg2;

- mm2reg <= em2reg;

- mwmem <= ewmem;

- mrn <= ern2;

- maluOut <= aluOut2;

- mqb <= eqb2;

- end

- endmodule

- module DataMemory (maluOut, mqb, mwmem, dmOut);//wm, aludata_in, qbdata_in, dmdata_out)

- input [31:0] maluOut, mqb; //input alu output value, qb value

- input mwmem;//write memory

- output reg [31:0] dmOut;

- reg [31:0] DM [0:36];

- initial begin //set first 10 words to data memory

- DM[32'd0] = 32'hA00000AA;

- DM[32'd4] = 32'h10000011;

- DM[32'd8] = 32'h20000022;

- DM[32'd12] = 32'h30000033;

- DM[32'd16] = 32'h40000044;

- DM[32'd20] = 32'h50000055;

- DM[32'd24] = 32'h60000066;

- DM[32'd28] = 32'h70000077;

- DM[32'd32] = 32'h80000088;

- DM[32'd36] = 32'h90000099;

- end

- always @ (maluOut, mqb) begin

- case (mwmem)

- 1'b0: dmOut <= DM[maluOut];

- 1'b1: dmOut <= DM[mqb];

- endcase

- end

- endmodule

- module MEMWB(clk, mwreg, mm2reg, mrn, maluOut, dmOut,

- wwreg, wm2reg, wrn, waluOut, wdmOut);//adding WB outputs

- input clk, mwreg, mm2reg;

- input [31:0] maluOut, dmOut;

- input [4:0] mrn;

- reg mwreg2, mm2reg2;

- reg [31:0] maluOut2, dmOut2;

- reg [4:0] mrn2;

- output reg wwreg, wm2reg;

- output reg [31:0] waluOut, wdmOut;

- output reg [4:0] wrn;

- always @(posedge clk) begin //stores into temporary regs

- mwreg2 <= mwreg;

- mm2reg2 <= mm2reg;

- maluOut2 <= maluOut;

- dmOut2 <= dmOut;

- mrn2 <= mrn;

- end

- always @(negedge clk) begin

- wwreg <= mwreg2;

- wm2reg <= mm2reg2;

- waluOut <= maluOut2;

- wdmOut <= dmOut2;

- wrn <= mrn2;

- end

- endmodule

- module WBMux(waluOut, wdmOut, wm2reg, wbMuxOut);

- input [31:0] waluOut, wdmOut;//takes in aluresult and data memory address

- input wm2reg; //write enable memory to register

- output reg [31:0] wbMuxOut; //output to regfile

- always @(waluOut, wdmOut, wm2reg) begin

- case(wm2reg)

- 0: wbMuxOut <= waluOut; //if wm2reg=0, output alu value

- 1: wbMuxOut <= wdmOut; //if wm2reg=1, output datamem value.

- endcase

- end

- endmodule

- module RegFile(clk, rs, rt, qa, qb, wrn, wbmuxOut); //(clk,rs,rt,qa,qb,wrn,wbmuxOut)

- input clk;

- input [31:0] wbmuxOut; //data to write to register

- input [4:0] rs, rt, wrn; //rs val in, rt val in, rd val in

- reg [31:0] regs [0:31]; //32 x 32 register file. Store all the registers.

- output reg [31:0] qa, qb; //qa->rt out, qb->rs

- initial begin //initialize all 32 registers to 0.

- {regs[0], regs[1], regs[2], regs[3], regs[4], regs[5], regs[6], regs[7],

- regs[8], regs[9], regs[10], regs[11], regs[12], regs[13], regs[14], regs[15],

- regs[16], regs[17], regs[18], regs[19], regs[20], regs[21], regs[22], regs[23],

- regs[24], regs[25], regs[26], regs[27], regs[28], regs[29], regs[30], regs[31]} = 0;//32'h00000000;

- end

- always @(posedge clk) begin //write during first half of cycle

- if(wrn != 0) begin//write operation

- regs[wrn] <= wbmuxOut;

- end

- end

- always @(rs, rt) begin//read during second half of cycle

- qa <= regs[rs]; //register val in register rs

- qb <= regs[rt]; //register val in register rt

- end

- endmodule

Testbench Module:

- //Taylan Unal CMPEN 331.001 Lab5 Honors Option Testbench

- `timescale 1ns / 1ps

- module CPU_Test();

- //WIRES ARE OUTPUTS, REGS ARE INPUTS.

- reg clk; //global clock signal

- //IF STAGE

- wire [31:0] pcin; //PC output to PC register

- wire [31:0] pcout; //PC output to adder

- wire [31:0] do; //instruction to be executed (im_out)

- //ID STAGE

- wire [5:0] op, func; //opcode and func fields.

- wire [4:0] rd,rs,rt; //source, destination registers of instruction

- wire [15:0] imm; //immediate field (for i-type instructions)

- wire [3:0] aluc; //alucontrol to input into ALU to determine operation

- wire wreg; //(ID) write enable for IDEXE register

- wire m2reg; //(ID) write enable for IDEXE register from data memory

- wire wmem; //(ID) write enable for data memory

- wire aluimm; //(ID) sends either alu control signal or immediate value to ALUMUX

- wire regrt; // (ID) input to controlMux, chooses what to write rt/rd to IDEXEregister

- wire [4:0] rn; //(ID) ** output of controlMux, chooses to write reg number to IDEXE

- wire [31:0] qa,qb;//(ID) ** register file values for instruction. (Note register is now at end of WB)

- wire [31:0] immExt; // (ID)extended immediate value to 32 bits (I-type)

- //EXE STAGE

- wire [3:0] ealuc; //(EXE) ALUControl to determine ALU operation

- wire ewreg; //(EXE) write enable for EXEMEM register

- wire em2reg; //(EXE) write enable for EXEMEM register from data memory

- wire ewmem; //(EXE) write enable for data memory

- wire ealuimm; //(EXE) sends either ALUcontrol signal or immediate value to ALUMUX

- wire [4:0] ern; //(EXE) **output of controlMux, chooses to write reg number to EXEMEM

- wire [31:0] eqa, eqb; //(EXE) **register file values for instruction

- wire [31:0] eimmExt; //(EXE) extended immediate value after written to IDEXE

- wire [31:0] muxImm; //(EXE) output of immediate mux

- wire [31:0] aluOut; //(EXE) output of ALU Module

- //MEM STAGE

- wire mwreg; //(MEM) write enable for MEMWB register

- wire mm2reg; //(MEM) write enable for MEMWB register from data memory

- wire mwmem; //(MEM) write enable for data memory

- wire [4:0] mrn; //(MEM) **output of controlMux, chooses to write reg number to MEMWB

- wire [31:0] maluOut; //(MEM) output of ALU Module

- wire [31:0] mqb; //(MEM) qb value from regfile->IDEXEReg->EXEMEMReg

- wire [31:0] dmOut; //(MEM) data memory output

- //WB STAGE

- wire wwreg; //(WB) write enable for Regfile. (After all other stages enable values

- wire wm2reg; //(WB) write enable for Regfile from datamemory

- wire [4:0] wrn; //(WB) **output of controlMux, chooses to write reg number to Regfile

- wire [31:0] waluOut; //(WB) output of ALU Module

- wire [31:0] wdmOut; //(WB) data memory output

- wire [31:0] wbMuxOut;//(WB) output of Mem2Reg Mux for DataMem->Regfile

- //FORWARDING UNIT and MUXes (All in ID Stage)

- wire [1:0] fwda; //output signal from ControlUnit, input for ForwardAMux

- wire [31:0] fwdaOut; //output signal from ForwardAMux

- wire [1:0] fwdb; //output signal from ControlUnit, input for ForwardBMux

- wire [31:0] fwdbOut; //output signal from ForwardBMux

- //////////////////////////////////////////////////////////////////////////////////////

- //START Testbench

- //IF STAGE

- PC PC(clk, pcin, pcout); //PCRegister: input clk, PCinput, outputs result from PCAdder

- PCAdder adder(pcout, pcin); //PCAdder: input PCOutput, increments by 4

- InstMem instmem(pcout, do); //InstMem: has instructions/outputs from PCRegister

- //IFID REGISTER

- IFID ifid(clk,do,//inputs

- op,rd,rs,rt,func,imm);//outputs

- //ID STAGE

- ControlUnit CU(op,func,rs,rt,mrn,mm2reg,mwreg,ern,em2reg,ewreg, //inputs

- aluc,wreg,m2reg,wmem,aluimm,fwda,fwdb,wpcir,regrt);//outputs

- ControlMux ctrmux(rd,rt,regrt,rn); //ControlMux: Selects whether to write rd/rt into RegWrite, outputs to IDEXE

- SignExtend extender(imm, immExt); //SignExtend: input is 16bit nonextended from instruction memory, output 32bit extended value.

- ForwardAMux fwdamux(fwda,qa,maluOut,mqb,dmOut,fwdaOut); //inputs: fwdb,qb,maluOut,mqb,dmOut outputs: fwda_out

- ForwardBMux fwdbmux(fwdb,qb,maluOut,mqb,dmOut,fwdbOut); //inputs: fwdb,qb,maluOut,mqb,dmOut outputs: fwdb_out

- //IDEXE REGISTER

- IDEXE idexe(clk,wreg,m2reg,wmem,aluc,aluimm,rn,qa,qb,immExt,//inputs

- ewreg,em2reg,ewmem,ealuc,ealuimm,ern,eqa,eqb,eimmExt);//outputs

- //EXE STAGE

- ALUMux alumux(eqb,eimmExt,ealuimm,muxImm);//ALUMux: selects qb reg output or immediate value

- ALU alu(eqa, muxImm, ealuc, aluOut);//ALU: executes operations for CPU using aluc control, a,b inputs

- //EXEMEM REGISTER

- EXEMEM exemem(clk,ewreg,em2reg,ewmem,ern,aluOut,eqb,//inputs

- mwreg,mm2reg,mwmem,mrn,maluOut,mqb);//outputs

- //MEM STAGE

- DataMemory datamem(maluOut,mqb,mwmem,dmOut); //DataMemory: data memory handler, inputs from EXEMEM, out to dmOut

- //MEMWB REGISTER

- MEMWB memwb(clk,mwreg,mm2reg,mrn,maluOut,dmOut,//inputs

- wwreg,wm2reg,wrn,waluOut,wdmOut); //ouputs

- //WB STAGE

- WBMux wbmux(waluOut, wdmOut, wm2reg, wbMuxOut); //WBMux: selects what to write back into regfile

- RegFile regfile(clk,rs,rt,qa,qb,wrn,wbMuxOut); //RegFile: generates a 32x32 register file to read/write from

- initial begin //Clock signal loop.

- clk = 0;

- end

- always begin

- #5 clk = !clk;

- end

- endmodule

Waveforms:

Design Schematic:

Design Schematic (contd):

Design Schematic (contd):

IO Planning:

Floor Planning: